EXAM 5 – DSIT 100

1. What are the finish classification requirements in Table 803.9 based on:
a. Occupancy type and sprinklers  b. Occupant load and location in building  c. Number of stories and sprinklers d. Building area and occupant load
2. What is the current California Building Code:
a. 2010 California Code of Regulations Part 24 

b. 2010 California Code of Regulations Part 2


c.     2010 California Code of Regulations Title 24 Part 2

d. 2010 California Standards Title 24 Part 2
3. Which assembly spaces are NEVER considered A Occupancy:
a. Those located building with B occupancy as main occupancy type
b. Those with occupant load of 300 or less

c.      Those with occupant load of 50 or less  



d. Those located in sprinklered building
4. What is building area as defined in Chapter 5:
a.    Area inside exterior walls
b. Area within exterior walls
c. Actual useable area
d. Area within exterior walls excluding shafts
5. Which of these is not a material testing system allowed by Chapter 8 Interior Finishes:
a. NFPA 286
b. NFPA 13D
c.  NFPA 265
d.  ASTM E 84
6. Type F Occupancies are divided in which types:
a. Over 300 occupants and under 300 occupants


b. High hazard or low hazard
   
c.     Over 50 occupants and under 50 occupants


d. Low hazard or medium hazard
7. Fire extinguishers are required to be located:
a. Near the exit    

b. In every assembly area 

 c. In a conspicuous location

d. In stairways
8. Exit passageway is a portion of:
a. Exit 
access
b. Exit
 c. Exit discharge
d. Corridor
9. Mixed occupancies are allowed under accessory occupancy provision when the accessory occupancy is:
a. 10% of total building area
b. 10% of main occupancy
c. 10% of actual useable area

d. 10% of rentable area
10. Primary concern of Chapter 8 Interior Finishes are what material characteristics:
a. Passive fire resistance
b. Whether they will ignite or burn
c. Fire and smoke resistance
d. Flame-spread and smoke generation
11. In which order would you consult the chapters of 2010 on a TYPICAL project:
a. Chapters 3, 5, 6, 10

b. Chapters 3, 4, 5, 10

c. Chapters 3, 5, 7, 10
d. Chapters 3, 5, 10, 11 (A or B)

12. Addition of sprinklers can modify which building characteristics:

a. Building area and occupant load
b. Building area only
c. Building area and height
d. Building area and construction type
13. Which of these types of construction is required to have sprinklers:
a. Type I 

b.  Type II

c. Type III

d. Type V
14. Means of egress is composed of which parts:
a. Exit access, exit, and exit discharge


b. Exit access, path of travel, exit discharge  

c.    Exit access, exit enclosure, exit discharge
 
d. Common path of travel, exit access, exit, exit discharge
15. Which of these fire protection systems are required to be shown on the architectural drawings:

a. Fire sprinklers

b. Fire extinguishers

c. Fire Alarms

d. Standpipe systems
16. Which is the only type of building occupancy that allows restraint of occupants:
a. I-1


b. I-2

c.  I-3

d. I-4
17. Fire barriers are NOT used to separate:
a. Exit enclosures
 

 b. Corridors     

 c. Separated occupancies

d. Fire areas
18. Portion of means of egress that has to be separated and protected by fire-resistance rated construction and opening protectives is :
a. Exit access
   b. Exit

 c. Exit Discharge
   
d. Corridor
19. Which of these buildings would not be considered R occupancy:
a. Apartment home    

b. Fraternity

    c.  Child care facility

d. Hospital
20. Which standard governs sprinkler systems:
a. NFPA 83  
b. UL723

c. NFPA 13
 d. ASTM E 84
