EXAM 6 – DSIT 100

1. When bathing facilities are provided for the public, clients or employees, what minimum percentage (not less than one) shall be accessible:

a.  1 percent  
b. 2 percent
c. 5 percent
 d. 10 percent
2. Accessible door hardware shall be (select best answer):
a. Of the lever type only



b. Mounted at 36” above finished floor


c.     Not require tight grasping, pinching or twisting of the wrist
d. Easy to operate with one hand and not require tight grasping
3. Entrances shall be connected by an accessible route to (select best answer):
a. Accessible parking and loading zones



b. Public way

c.      Accessible parking, public transportation stops and public sidewalks
d. Accessible parking and public way
4. Loading, drop off, and parking access aisles shall not exceed surface slope of:
a.    1 percent

b. 2 percent
c. 3 percent
d. 5 percent
5. Interruption of a curb at pedestrian way which separates surfaces that are substantially at the same elevation is:
a. Curb cut

b. Curb ramp
c.  Pedestrian Ramp
d.  Pedestrian Way
6. Accessible route is required to provide which for persons with disabilities:
a. Entrance and exit from the building and access to facilities

b. Egress from the building
c.     Access to all facilities




d. Access to facilities from the public way
7. Maximum accessible height of side reach is:
a. 36”    

b. 48” 

 c. 54”

d. 60”
8. Cross-slopes for accessible walking surfaces shall not exceed:
a. 1 percent

b. 2 percent
 c. 5 percent
d. 8 percent
9. Dining, banquet and bar areas shall have one wheelchair space for each:
a. 10 seats

b. 20 seats
c. 50 seats
d. 100 seats
10. What is the minimum width of access aisle at accessible parking:
a. 3 feet
b. 4 feet

c. 5 feet

d. 8 feet
11. Minimum accessible door or aisle width is:
a. 30 inches

b. 32 inches

c. 36 inches
d. 42 inches

12. Unisex accessible toilet facilities can be provided:

a. When separate accessible facilities are not available 
b. When client wants to avoid modifying existing facilities

c. When facilities are not open to the public

d. When other provided facilities are unisex
13. Accessible section of counter shall be at least how long:
a.   30” 

b.  36”

c. 48”

d. 60”
14. Define “equal facilitation” for persons with disabilities:
a. Access to facilities as long as it does not impair comfort of others
b. Maximum independence no matter the impact on others  

c.    Maximum independence with least risk of harm to others
 
d. Identical facilities for everyone
15. Maximum accessible height of front reach is:

a.   36”

b. 48”


c. 54”


d. 60”
16. Minimum dimensions of accessible passing space in corridors is:
a. 30” x 60”

b. 48” x 60”

c.  60” x 60”

d. 72” x 60”
17. Accessible parking shall be provided for:
a. Clients and guests 
 b. For the public, unless valet is provided      c. For the employees
d. For the public, including employees
18. Pedestrian aisles in M occupancies shall be minimum of what clear width :
a. 32”
  

b. 36”

 c. 44”  

d.  48”
19. Detectable warnings are required (select best answer):
a. At accessible parking     b. When walk crosses vehicular way
   c.  At supermarkets    d. When walk crosses or adjoins vehicular way
20. Describe accessibility in your own words:
