EXAM 8 – DSIT 100

1. Who holds the liability for enforcing building codes?

a. Building Official
b. Contractor 
c. Designer
d. Structural Engineer

2. What is the definition of Interior Designer?

a.  Well-informed practitioner making decisions any other design professional would in the same situation.

b. Professional design practitioner enforcing the Building Code.

c.  Professional design practitioner qualified by education, experience and examination to protect health, life safety and welfare of the public.

d. Design professional acting the in best interests of the public and carrying the liability for enforcing the Building code.

3.  What is the full name of current Energy Code in California:
a.    2010 CCR Title 24 Part 6

b. Title 24

c.   2010 CALGreen

d. 2010 CCR Title 24 Part 2
4.  Building Area is defined as:
a. Area inside surrounding exterior walls  b. Area within surrounding exterior walls  c. Area excluding surrounding exterior walls  d. Area of all floors
5.  Non-Combustible material:
a. … will not ignite or burn 
b.  … will not burn

c. … is fire resistant 
d. … has passive fire resistance
6. Which one of these is a characteristic of ALL I Occupancies:
a. Non-ambulatory occupants
b. Restraint
c. Supervised environment
d. More than 50 occupants
7. Which code actually contains California’s energy efficiency provisions:
a.    2010 CALGreen
b. 2010 California Building Code, Chapter 13
 c. 2010 California Energy Code
d. 2009 LEED CI
8. Corridor is considered a portion of:
a. Exit access
b. Exit
c.     Exit Load  
d. Exit Discharge
9. Occupant load is:

a.  Maximum number of occupants that will fit within an area


b. Determined based on occupancy type

c. Maximum number of occupants client requested to have

d. Number of persons for which the means of egress is designed
10. Common path of egress travel is:
a. Portion of exit access occupants are required to transverse before two separate paths to two exits are available

b. Maximum travel distance from most remote point within a story along unobstructed path of egress to an exit

c. Point at which two separate and distinct paths to two exits become available

d. Commonly used path of travel that is part of means of egress
11. Egress width is based on:
a. Total floor area  

b. Occupancy 

 c. Occupant Load 

 d. Building height
12. Egress door minimum clear width is:

a. 30”


b. 32”

c.36”

d. 44”
13.  Guards are required if there is a drop from a walking surface of at least:
a. 12”    


b. 24”

 c.  30”

d. 36”
14.  Fire extinguishers are required to be located:
a. Near the exit    

b. In every assembly area 

 c. In a conspicuous location

d. In stairways
15.  Primary concern of Chapter 8 Interior Finishes are what material characteristics:
a. Passive fire resistance
b. Whether they will ignite or burn
c. Fire and smoke resistance
d. Flame-spread and smoke generation
16. In which order would you consult the chapters of 2010 on a TYPICAL project:

a. Chapters 3, 5, 6, 10

b. Chapters 3, 4, 5, 10

c. Chapters 3, 5, 7, 10
d. Chapters 3, 5, 10, 11 (A or B)
17. Dining, banquet and bar areas shall have one wheelchair space for each:

a. 10 seats

b. 20 seats
c. 50 seats
d. 100 seats

18. What is the minimum width of access aisle at accessible parking:

a. 3 feet
b. 4 feet

c. 5 feet

d. 8 feet

19.  What is Americans with Disabilities Act:
a. Part of 2010 California Building Code     b. Model code
   c.  Federal law (not code)    d. Part of 2010 CCR Title 24
20. Describe the intent of accessiblity in your own words:
21. Describe “standard of care” for Interior Designer in your own words:
